# 3 · Program logic reference (the exact PLC code)

*This document is the map of the real safety program. It tells you, page by page, where each safety function lives in program.pdf and what its logic actually does — so when something trips in offline simulation you can find the exact block that decided it.*

*This is the authoritative map of program.pdf (713 pages, HIMA SILworX export, SILworX v16, project C15862H-BL10E, configuration BL10E_HeXI [400], target HIMax, SIL 2). The searchable text is in data/program_extracted.txt (page markers ===== PAGE n =====).*

A few words before you start (plain English):

A SIF is one independent safety function — one self-contained job the PLC

does to keep people safe (for example "is the door shut?").

2oo3 voting means 2 of 3 sensors must agree before the PLC believes

them. It tolerates one faulty or disagreeing sensor.

An RS latch is a memory bit that, once set, **stays set until something

resets it** — so a trip does not clear on its own.

A TON is an on-delay timer: the output turns on only after the input

has stayed on for a set time.

An SFC is a step-by-step state machine — the program walks through a

fixed list of steps in order (used for the hutch search).

A permissive is a signal that is TRUE when everything is healthy / OK

and drops to FALSE to trip.

De-energise-to-trip means the safe state is OFF. If power is lost the

device falls to its safe position by itself.

Convention in SILworX FBD text: & = AND, >=1 = OR, NOT/a bubble = invert. All effects are de-energise-to-trip: an output FALSE = tripped/safe; a SIF ..._OUTPUT/..._TRIP signal TRUE usually means healthy/permissive.

__________________________________________________

## 3.1 Page index (where everything lives)

| Pages | Area |
| --- | --- |
| 1–13 | Cover + Table of Contents |
| 14–16 | Project structure (POU tree, lists all SIFs) |
| 37–126 | Global Variables & cross-references (the master tag list + initial values, setpoints, timer presets) |
| 127–149 | Hardware (HIMax I/O module config) |
| 151–285 | C15862H_Lib — custom application logic (see 3.4) |
| 286–382 | The 12 SIFs (core safety logic) |
| 383–690 | X_Lib — HIMA standard blocks (voters, timers, limits) |
| 691–710 | Modbus TCP / OPC-UA / safeethernet |
| 711–713 | Watchpages (Global Forcing) |

## 3.2 The 12 SIFs (page → logic)

Reminder: a SIF is one independent safety function — one self-contained safety job.

| SIF | Pages | Logic pg | Title |
| --- | --- | --- | --- |
| SIF-01 | 287–291 | 291 | General Access Door Closed (main personnel door is shut) |
| SIF-02 | 292–297 | 297 | Service Access Door Closed (maintenance door is shut) |
| SIF-03 | 298–303 | 303 | Current & Voltage Monitoring (is the source really off?) |
| SIF-04 | 304–338 | 326–338 | Search Sequence (SFC — the step-by-step hutch search) |
| SIF-05 | 339–344 | 344 | Search Start (may a search begin?) |
| SIF-06 | 345–350 | 350 | Search Complete (all doors proven shut) |
| SIF-07 | 351–356 | 356 | Beam On (permissive — TRUE = OK for beam) / lock feedback |
| SIF-08 | 357–363 | 363 | Beam Off Buttons (the emergency-stop buttons) |
| SIF-09 | 364–368 | 368 | Radiation Monitoring |
| SIF-10 | 369–372 | 372 | Electron Source Enable (the enable key) |
| SIF-11 | 373–377 | 377 | Oxygen Monitoring |
| SIF-13 | 378–382 | 382 | General Access Gate Closed (the entrance gate is shut) |

*(No SIF-12 in the PLC — the C&E "SIF 11 & 12" both refer to the oxygen function.)*

## 3.3 Per-SIF logic (plain English)

SIF-01 (p.291) — GADC-01/02/03 (three "closed" switches on the **main

personnel door) voted 2oo3 (2 of 3 must agree) using `X_2oo3_B` (HIMA's   standard 2oo3 voter block), AND'd with `SIF-07_OUTPUT` (the beam-on master   permissive: TRUE = beam allowed). Output `SIF-01_TRIP` (door function — TRUE   = door shut, OK to run) → contactors CON-01/02/03 (power contactors feeding the   electron source**) via the central Logic POU.

SIF-02 (p.297) — SADC-01/02/03 (three "closed" switches on the **service /

maintenance door) voted 2oo3 OR** SADC-04/05 (two more service-door   switches, a 1oo2 pair — either one is enough), AND'd with SIF-07_OUTPUT   (beam-on permissive). Outputs SIF-02_TRIP (service-door function — TRUE =   service door shut, OK) and SERVICE_DOOR_CLOSED.

SIF-03 (p.303) — "is the beam actually off?" + door-unlock permit. Current

IT-01/02/03 (current transmitters on the electron source — a high reading   means the source is live) voted 2oo3, and Voltage VT-01/02/03 (voltage   transmitters — high = source live) voted 2oo3. An RS latch (a memory that   stays set until reset) + a TON (on-delay timer) DOOR_UNLOCK_DELAY = T#20s   + an R_TRIG (rising-edge detector) on the Open/Reset pushbutton OR-01 (the   Open / Reset button the operator presses to unlock and reset) and key   KEY-01 (the electron-source enable key at the control panel): the   Open/Reset button only becomes live once both 2oo3 votes read zero for   ≥20 s, then OR-01:LED lights. (The C&E footnote mentions 12 s from the   SRS; the implemented value is 20 s.)

SIF-04 (pp.326–338) — the SFC (the step-by-step state machine for the

search; see 3.5).

SIF-05 (p.344) — Search-start permit (T1): SCR-01 (the **search card

reader at the door) AND `KEY-01` (control-panel enable key) AND `KEY-02` (the   hutch enable key, needed to start a search) AND service-door 2oo3 AND the   1oo2 pair AND `SIF-08` healthy (no beam-off button pressed) → `SIF-05_START`   (the search-start permit** — TRUE = a search may begin).

SIF-06 (p.350) — Search-complete: general door 2oo3 AND gate 2oo3 →

SIF-06_TRIP (search-complete proof — TRUE = all doors and the gate are   shut, so the search may finish; the T7 proof that every door is shut).

SIF-07 (p.356) — master beam-on permissive (TRUE = beam allowed):

door-lock confirmation switches GADL-01/02/04/05 (the "locked" switches that   feel the bolt of the main door / gate) + SADL-01/02 (the "locked"   switches on the service door) + SIF-06, with a TON (on-delay timer)   BEAM_DELAY_TIMER = T#180s and an RS latch (stays set until reset) →   SIF-07_OUTPUT. Consumed by SIF-01/02/13 and the SFC.

SIF-08 (p.363) — eight dual-channel beam-off buttons BOB-01…08 (the

Beam-Off Buttons / emergency stops; each has two wires :A and :B, AND'd   per button for safety), voted 1oo7 (any one pressed trips), RS-latched   (stays tripped until reset), reset by IOC-01-BOB:RESET (the control-room   reset button for the beam-off-button latch). Drops CON-01,02,03,06,07,08,   09,10 and instantly unlocks.

SIF-09 (p.368) — RDMND-01/02 (radiation monitors — accumulated

dose) + RDMNR-01/02 (radiation monitors — dose rate), voted 1oo2   (any one trips), RS-latched, reset by IOC-01-RDMN:RESET (control-room reset   button for the radiation latch). Drops CON-01/02/03 and CON-09.

SIF-10 (p.372) — KEY-03 (the electron-source enable key at the

equipment rack), 1oo1 → SIF-10_TRIP (enable-key function — TRUE = key on,   OK); gates CON-04/05 (and CON-09/10 RF). Combined with KEY-01 (the   control-panel ZCP enable key) elsewhere.

SIF-11 (p.377) — OXMON-01…04 (the four oxygen monitors in the hutch —

a low reading means the air is unsafe to breathe), voted 1oo4 (any one   trips), RS-latched, reset by IOC-01-GAS:RESET (control-room reset button for   the oxygen/gas latch). Closes LN₂ valve SDVLN2 (the liquid-nitrogen   shut-off valve), and switches O₂ beacons/indicators IND-01/02/03 (oxygen   traffic-light indicators — green = air OK, red = oxygen low) and speaker SP02.

SIF-13 (p.382) — gate GADC-04/05/06 (three "closed" switches on the

entrance gate) voted 2oo3, AND'd with SIF-07_OUTPUT (beam-on permissive)   → drives CON-06/07/08 (the 3-phase RF power contactors) with a 200 ms   delay. Output SIF-13_TRIP (gate function — TRUE = gate shut, OK). (Added in   Rev 4.0 to split gate from door.)

## 3.4 The central `Logic` POU (pp.267–269) — cause → effect

This is the block that actually drives the outputs:

Contactors (p.267): combine the SIF trips per group and RS-latch them

(latch = a memory that stays set until reset) so that CON-01,02,03,06,07,08,   09,10 (the electron-source and RF power contactors) cannot re-energise after a   trip without a fresh full search (the SEARCHED_AND_LOCKED latch — the memory   bit meaning "search finished and doors locked"). This is why, after any door or   beam-off-button trip, you must run a fresh search before the hazard can come   back. Recall de-energise-to-trip: a contactor at 0 is the safe (isolated)   state.

Door/gate solenoids `SOL-01…06` (the lock bolts on the doors/gate): service

doors lock when the search *starts*; general doors/gate lock when the *final*   button is pressed; all unlock when status = Open or SIF-08 (a beam-off button)   is active.

Lights/annunciators/sounders (pp.268–269): white light vs blue light

interlock, annunciator OPEN/RESTRICTED/STANDBY/BEAM-ON (the lit hutch signs),   key solenoids, speaker programs, O₂ beacon.

## 3.5 The Search SFC + Transition_Timer (SIF-04)

An SFC is a step-by-step state machine: the program walks through a fixed list of steps in order, and each step must be reached the right way before the next is allowed.

Layer 1 — the SFC steps (Modbus bit in brackets): Open (0) → Open-Ready (1) → Search_Start (2) → Hutch_Entered (3) → ASB1 (4) → ASB2 (5) → ASB3 (6) → ASB4 (7) → Standby (8) → Beam On (9); with abort branches Search_Aborted, Start_Aborted, Run_Aborted. (These match the STATUS-… search-state tags: STATUS-OPEN, STATUS-OPEN_READY, STATUS-START_SEARCH, STATUS-HUTCH_ENTERED, STATUS-ASB1…ASB4, STATUS-STANDBY, STATUS-BEAM_ON.) Modbus bit 10 = SEARCHED_AND_LOCKED — the "search finished and doors locked" memory bit, a convenient state handle for an HMI.

Layer 2 — six `Transition_Timer` instances (custom FB defined p.285), one per gap T1-T2 … T6-T7. Each takes Start_Timer, Min_Time, Max_Time, Reset and outputs Below_Min_Time (→ _TRIP_LOW: the step was done too fast), Exceed_Max_Time (→ _TRIP_HIGH: the step took too long) and Transition_Time (ms). All six use `Min = T#5s`, `Max = T#60s` and are reset by STATUS-START_SEARCH (the tag meaning a search has begun).

## 3.6 Verified timing constants (Global Variables)

| Tag | Value | Purpose |
| --- | --- | --- |
| MAX_SEARCH_TIME | 180 s | overall search watchdog (whole search must finish in time) |
| T1-T2…T6-T7_TIMER_MIN | 5 s | min per-step time — a step done faster ⇒ abort |
| T1-T2…T6-T7_TIMER_MAX | 60 s | max per-step time — a step slower ⇒ abort |
| BEAM_DELAY_TIMER_TIME | 180 s | Standby → Beam-On dwell (the warning wait before beam) |
| RESET_DELAY_TIMER | 60 s | delay before returning to the Open state |
| DOOR_UNLOCK_DELAY | 20 s | SIF-03 zero-source confirm (source must read off this long) |
| LIGHT_CURTAIN_DELAY | 2 s | light-curtain monitoring arm delay |
| gate/RF contactor delay | 200 ms | CON-06…10 (RF contactors) trip delay |

## 3.7 Function-block types used

Voters: X_2oo3_B (HIMA's standard 2oo3 voter — sum of 3 BOOLs ≥ 2, so 2

of 3 must agree; also gives a discrepancy Dev output) — used in   SIF-01/02/03/05/06/13. The higher votes 1oo4/1oo7/1oo2 in SIF-08/09/11 are done   with plain &/>=1 gating + RS latches (memory bits that stay set until   reset), not voter blocks.

Analogue trips: X_LimH (HIMA's standard high-limit trip block) and

X_LimL (the low-limit trip block), each with hysteresis (a small dead-band   so the trip does not flicker), trip-delay DT, Inhibit, Reset, Ch_ok.   X_Hx_AI scales raw HIMax counts to engineering units.

IEC blocks: TON (on-delay timer — output turns on only after the input has

stayed on for the set time), TP (pulse), RS/SR (latches — memories that   stay set until reset), R_TRIG/F_TRIG (edge detect — fire on a 0→1 or 1→0   change), MOVE, LIMIT, comparators.

Custom: Transition_Timer, 2In_/3In_Discrepency_Alarm.

## 3.8 Analogue setpoints (as exported)

Each of these sensors makes a _TRIP flag inside the PLC: 1 = the limit has been crossed (oxygen too low, or source live), 0 = normal. The _SP tag is the set-point (the level at which it trips) and _HYST is the hysteresis (the small dead-band).

| Signal | Block | Setpoint (_SP) | Hyst (_HYST) |
| --- | --- | --- | --- |
| OXMON-01…04 (O₂ monitors — low reading = unsafe air) | X_LimL (low-limit trip) | 25.0 (full-scale placeholder; commissioned value ≈ 19.5 %) | 0.2 |
| IT-01…03 (current transmitters — high = source live) | X_LimH (high-limit trip) | 0.0 (placeholder) | 0.2 |
| VT-01…03 (voltage transmitters — high = source live) | X_LimH (high-limit trip) | 0.0 (placeholder) | 0.2 |
| RDMN dose/rate (radiation monitors) | digital trip contacts (>4 mA per C&E) | — | — |

Each AI channel also exposes _OC (open-circuit), _SC (short-circuit) and _CH_OK (channel-OK) wiring-health diagnostics. The placeholder setpoints (0.0 on current/voltage, 25.0 on oxygen) are what the program exports today; the commissioned trip points (≈ 19.5 % O₂, and ~4 mA on radiation) are set at site — see document 04 (the SILworX offline test procedure) for how to confirm them.

## 3.9 Things worth knowing for support work

Latching / "no silent restart": trips and the search-complete state are

RS latches (memory bits that stay set until reset). After any trip you need   an explicit reset *and* (for the hazard) a fresh search before the beam   can return.

Resets are separate: IOC-01-BOB:RESET (resets SIF-08, the beam-off

buttons), …-RDMN:RESET (resets SIF-09, radiation), …-GAS:RESET (resets   SIF-11, oxygen), and the Open/Reset button OR-01 (SIF-03 — only live after the   20 s zero-source confirm, so watch OR-01:LED). Each is its own control-room   button.

Dual-channel devices (the beam-off buttons BOB, blue lights BLUEL,

annunciator signs ANNUN, beam-on indicator BONI, beam-off indicators BOBI)   are read as :A/:B pairs and AND'd; discrepancy-alarm blocks give first-up   annunciation if the two channels disagree.

No application bypass/maintenance-override block exists — overrides are done

only via SILworX Global Forcing (Watchpages pp.711–712). (If you saw an   LXS_BYPASS block elsewhere, that belongs to a *different, unrelated* project   that used to be in this repo's history — not B10E.)

Comms: Modbus TCP slave on port 502 exposes the SFC state bits and the

oxygen readings; an OPC-UA server mirrors most tags. Good HMI/SCADA handles.

Naming wart: the PLC tag database uses BL10E-PS-… while the SIF block

names and the C&E "EPICS/DCS Tag" use B10E-PS-…. Same devices. Door locks   appear twice: as outputs SOL-01…06 (the lock bolts) and as feedback inputs   GADL-/SADL- (the "locked" switches that feel those bolts).

*See also: document 00 (tag glossary — every signal name), document 01 (the system overview), document 02 (the hutch search explained), and document 04 (the SILworX offline test procedure — how to exercise this logic in offline simulation).*

*External drawings referenced by the C&E but not in this repo:* SRS TDI-PSS-SRS-0002, SIF logic Dwg 1224211, layout Dwg 1212854.