# 6 · SILworX offline-simulation test procedure — v2.2 (complete, with concepts)

*Run this in HIMA SILworX offline simulation against the actual B10E program. For every set of logic it gives you: the Concept (what it protects and why), the Logic as built (the real blocks/signals on the named page), the Test (exact forces and expected observations), and What it proves. Follow it top to bottom and you will have exercised — and understood — the whole program.*

Source of truth: program.pdf (project C15862H-BL10E, configuration BL10E_HeXI [400], SILworX v16.0.0 R3326). Page text: data/program_extracted.txt. Changes from v1: added SIF-05 negative tests, a standalone SIF-07 test, the lock-feedback forcing step (without it the search stalls at STANDBY in offline sim), voltage (VT) tests, verified contactor groupings from p.267, dual-channel discrepancy tests, the KEY-01 release test, and a coverage matrix. v2.1: corrected analogue handling — the _SP setpoints are fail-safe constants (OXMON 25.0 / IT,VT 0.0) that cannot be forced; baseline and SIF-03/-11 tests now force the _TRIP flags (Option A) or require a setpoint edit + code regen (Option B). v2.2: added a plain-language "what the tag is" column to the §0.3 polarity table, and a high-school-level explanation of every timer in §0.5.

__________________________________________________

## 0 · Setup & conventions (read once, it saves hours)

### 0.1 Start offline simulation

Open the project; select Resource `BL10E_HeXI [400]`.

Generate code, then start Offline Simulation on the resource.

Open any logic page (e.g. Library/SIF/B10E-SIF-01) — wires show live values.

Open the Force Editor / a Watch page; add the tags listed in each test.

### 0.2 Concept: forcing replaces the field

In offline simulation there is no I/O. The global variables that normally come from field wiring (BL10E-PS-…) just sit at their initial values — you are the field. Forcing an input variable is "operating the plant". Two traps:

Feedback inputs do not follow outputs. When the program energises a door

lock output (SOL-01), the lock position feedback input (GADL-01) does   NOT change by itself — in the real plant the mechanism moves the switch; in   simulation you must force the feedback to simulate the mechanism. This   matters at T7 (§3) — if you don't force GADL/SADL TRUE after the locks   energise, BL10E-SIF-07_OUTPUT never comes TRUE and beam-on never happens.   The same idea applies to contactor feedbacks (CON-0x:FB) used by the alarm   layer.

Polarity. This program is de-energise-to-trip throughout, so for most

inputs the demand (unsafe) value is 0, and many …_TRIP/…_OUTPUT   signals are permissives: TRUE = healthy, FALSE = tripped. Exception: the   analogue …_TRIP flags (IT/VT/OXMON) are detections: TRUE = limit exceeded.

### 0.3 Polarity table (force the right way)

| Signal | What the tag is (the device and its job) | Healthy / normal | Demand (unsafe / action) |
| --- | --- | --- | --- |
| BL10E-PS-GADC-01..06, SADC-01..05 (door closed sw.) | Position switches on each personnel door and gate — they tell the PLC whether the door is physically shut | 1 = closed | 0 = open |
| BL10E-PS-GADL-01/02/04/05, SADL-01/02 (lock feedback) | Switches on the lock bolts — they confirm a door is *actually* locked, not just that the PLC asked for it | 1 = locked-confirmed | 0 = not locked |
| BL10E-PS-KEY-01/02/03 (enable keys) | Physical key-switches the responsible person turns to allow a search / let the source be energised | 1 = on | 0 = off |
| BL10E-PS-SCR-01 (search card) | Card reader at the door — presenting a valid search card is what starts the search | 1 = card in | 0 = out |
| BL10E-PS-BOB-01..08 :A and :B (beam-off, dual ch.) | Emergency "beam-off" buttons in and around the hutch; each has two separate wired channels (:A, :B) for safety | 1 = not pressed | 0 = pressed |
| BL10E-PS-ASB-01..04, ASBF-01 (search buttons) | Area-search pushbuttons pressed in a set order during the walk-through; ASBF is the final one by the exit door | 0 = idle | pulse 0→1 = press |
| BL10E-PS-LCRx-01 (light curtain) | An invisible infra-red "curtain" across the entrance that detects a person stepping through | 1 = clear | 0 = interrupted |
| BL10E-PS-OR-01 (Open/Reset PB) | The "Open / Reset" button the operator presses to unlock the doors and reset the system once the source is proven off | 0 = idle | pulse 0→1 |
| BL10E-PS-IT/VT-0x_TRIP (BOOL — see §0.4!) | Trip flags from the electron-source current (IT) and voltage (VT) transmitters; 1 means the source is electrically live | force 0 = source dead | force 1 = current/voltage present |
| BL10E-PS-OXMON-0x_TRIP (BOOL — see §0.4!) | Trip flags from the four hutch oxygen monitors; 1 means oxygen has dropped too low (suffocation risk) | force 0 = O₂ healthy | force 1 = O₂ low |
| BL10E-PS-RDMND/RDMNR-01/02 (radiation) | The two radiation monitors: accumulated dose (RDMND) and dose rate (RDMNR) | healthy | trip value → SIF-09 |
| BL10E-PS-IOC-01-BOB/RDMN/GAS/SYS:RESET | Control-room reset buttons — one each for the beam-off, radiation, gas and system latches | 0 | pulse 0→1 = reset |
| OUTPUT BL10E-PS-CON-0x:EN | The PLC's command to a power-contactor coil that feeds the electron source / RF — i.e. switches the hazard ON | — | 1 = energised (hazard live), 0 = safe |
| OUTPUT BL10E-PS-SOL-01..06 (locks) | The PLC's command to a door/gate lock solenoid (the bolt that locks the door) | — | 1 = locked, 0 = unlocked |

### 0.4 Critical: the analogue setpoints are fail-safe CONSTANTS

The as-built values (Global Variables, verified in the export) are:

| Variable | Initial value | Block | Effect as exported |
| --- | --- | --- | --- |
| OXMON-01..04_SP | 25.0 | X_LimL (trips below) | top-of-range ⇒ oxygen trip always active |
| IT-01..03_SP, VT-01..03_SP | 0.0 | X_LimH (trips above) | zero ⇒ "source present" always active |
| all …_HYST | 0.2 | — | — |

Two consequences:

They are deliberately fail-safe placeholders — until commissioning sets

real values, the analogue functions sit in their tripped (safe) state.

They are declared `Constant` in the variable table — so in offline

simulation you cannot force `_SP`/`_HYST` at runtime. Forcing the field    value (e.g. IT-01 = 3.9) can therefore NEVER clear these trips (3.9 > 0.0).

Practical choices for offline simulation:

Option A (recommended, no code change): leave the values alone and

force the `_TRIP` BOOLs directly — OXMON-01..04_TRIP = 0,   IT-01..03_TRIP = 0, VT-01..03_TRIP = 0 for healthy; set individual ones   to 1 to simulate a trip. Forcing overrides the POU's write, so the SIF logic   downstream behaves exactly as designed. (You lose only the limit-block   internals, which Option B covers.)

Option B (to exercise the X_LimH/X_LimL blocks themselves): edit the

_SP initial values in the global-variable editor (e.g. OXMON_SP =   19.5, IT/VT_SP = 6.0), regenerate code, restart the simulation — then   field-value forcing works as in §2.1. Revert afterwards; on a real system   this is a controlled modification.

### 0.5 Concept: the timers run on the simulation clock

MAX_SEARCH_TIME = 180 s, six per-step windows 5 s…60 s, BEAM_DELAY = 180 s, DOOR_UNLOCK_DELAY = 20 s, LIGHT_CURTAIN_DELAY = 2 s, RESET_DELAY = 60 s, alarm dwell E_SOURCE_TIME = 120 s. In offline sim these elapse in (simulated) real time — between search steps wait ≥5 s and <60 s.

In plain words — what each timer is for. Think of each one as a stopwatch the safety PLC uses so things can't happen too fast or too slow:

| Timer | Value | What it does, in everyday terms |
| --- | --- | --- |
| MAX_SEARCH_TIME | 180 s (3 min) | The whole hutch search must be finished within 3 minutes, or it cancels and you start again. |
| per-step window | 5–60 s | Between each search button you must wait at least 5 seconds (so nobody can rush or "tailgate" the search) but no more than 60 seconds (so you can't wander off half-way). |
| BEAM_DELAY | 180 s (3 min) | Once the search is done, the system waits 3 minutes — with warning sirens and lights ("radiation imminent") — before it will switch the beam on, giving anyone still inside time to hit a beam-off button. |
| DOOR_UNLOCK_DELAY | 20 s | After the source reads zero, the PLC waits 20 seconds to be sure it is *really* off before it lets you unlock the doors. |
| LIGHT_CURTAIN_DELAY | 2 s | A short 2-second pause after someone steps through the doorway before the light curtain starts watching for extra people. |
| RESET_DELAY | 60 s (1 min) | A one-minute wait before the system settles back to the fully "Open" state. |
| E_SOURCE_TIME | 120 s (2 min) | How long the source must stay switched off before the alarm screen accepts it as truly dead. |

__________________________________________________

## 1 · Baseline (run before every test block)

Concept. Every test must start from a known-good plant: all doors shut, keys in, nothing pressed, process values healthy. The program shows this as STATUS-OPEN_READY = 1 ("ready to start a search").

Force TRUE (1): GADC-01..06, SADC-01..05, KEY-01, KEY-02, KEY-03, all sixteen BOB-0x:A/:B, LCRx-01. Force FALSE (0): ASB-01..04, ASBF-01, SCR-01, OR-01, GADL-01/02/04/05, SADL-01/02 (locks not yet engaged), radiation RDMND/RDMNR-01/02 healthy. Force the analogue trip flags FALSE (0) — per §0.4 Option A (the _SP constants make value-forcing ineffective): OXMON-01..04_TRIP = 0, IT-01..03_TRIP = 0, VT-01..03_TRIP = 0. *(Option B users: with edited setpoints, force IT/VT-0x = 3.9 and OXMON-0x = 20.9 instead.)* Pulse once: …-BOB:RESET, …-RDMN:RESET, …-GAS:RESET (clear power-up latches).

Expect: STATUS-OPEN_READY = 1; ANNOPN-01:A/:B = 1; every CON-0x:EN = 0; every SOL-0x = 0; all BL10E-SIF-0x_TRIP at their healthy value (mostly 1).

__________________________________________________

## 2 · Input-layer logic (how a field signal becomes a trip)

### 2.1 Concept: analogue channel → limit block → `_TRIP` flag

Each 4–20 mA input (oxygen, current, voltage) is scaled (X_Hx_AI, p.588) and fed to a limit block — X_LimH (p.507) trips above its setpoint (current, voltage), X_LimL (p.514) trips below it (oxygen). Each block has a setpoint …_SP, hysteresis …_HYST, a trip delay DT, and produces the BOOL …_TRIP. The channel also publishes _OC/_SC/_CH_OK (open/short-circuit, channel-OK) diagnostics.

This test requires §0.4 Option B (the _SP values are Constant — they cannot be forced). Edit the initial values in the global-variable editor (OXMON-01_SP = 19.5, IT-01_SP = 6.0 for this test), regenerate code and restart the simulation first. If you stay on Option A, skip Test 2.1 — the SIF-level behaviour is fully covered by forcing _TRIP in §3.

Test 2.1 — open Analogue Inputs logic (pp.184–187), with edited setpoints:

| Step | Force | Watch / expect |
| --- | --- | --- |
| 1 | OXMON-01 = 20.9 | OXMON-01_TRIP = 0 |
| 2 | OXMON-01 = 18.0 | OXMON-01_TRIP → 1 (low trip) |
| 3 | OXMON-01 = 19.6 (just above SP 19.5) | stays tripped until SP+HYST cleared — hysteresis |
| 4 | IT-01 = 12.0 | IT-01_TRIP → 1 (high trip) |

What it proves: you can trace any analogue from raw value to its _TRIP flag, and you understand setpoint + hysteresis.

### 2.2 Concept: dual-channel digital inputs and discrepancy

Safety pushbuttons (BOB) and indicators are two independent channels :A and :B, AND'd so a single wiring fault cannot fake a healthy button. A 2In_Discrepency_Alarm (def. pp.152–156; instantiated in Alarms, pp.171–174) raises an alarm if :A ≠ :B for longer than its delay — that is a fault, not a trip.

Test 2.2: force BOB-05:A = 0 while BOB-05:B = 1. Expect: SIF-08_TRIP behaviour per design (single-channel demand is still a demand — watch it live), and the BOB-05 discrepancy alarm in Alarms (p.171). Restore both to 1, reset. What it proves: channel-fault detection is separate from the trip function.

__________________________________________________

## 3 · The SIF tests (concept + logic + test, one by one)

Re-establish §1 between tests. Page references are the logic pages.

### SIF-01 — General Access Door (p.291)

Concept. If the main personnel door is open, the electron source must be isolated — radiation must never coexist with an open door. Three independent closed-switches on the door vote 2oo3: one broken/misaligned switch must not spuriously kill the beam (availability), two open switches must (safety). Logic as built. X_2oo3_B(GADC-01,02,03).Out & BL10E-SIF-07_OUTPUT → BL10E-SIF-01_TRIP (a permissive: 1 = healthy). The voter (p.644) sums the three BOOLs and outputs TRUE when ≥2 are TRUE; its Dev pin flags channel disagreement after delay DT. Test. From BEAM_ON (§4 walkthrough) or baseline:

| Step | Force | Expect |
| --- | --- | --- |
| 1 | GADC-01 = 0 | voter Out stays 1 — no trip (2oo3 tolerates one); after DT, voter Dev flags discrepancy |
| 2 | GADC-02 = 0 | voter Out → 0, SIF-01_TRIP → 0; `CON-01/02/03:EN → 0` (and CON-10:EN, same latch group, p.267) |
| 3 | restore both = 1 | contactors stay 0 — see §5 latch test |

### SIF-13 — General Access Gate (p.382)

Concept. Rev 4.0 split the gate from the door so each has its own function; the gate isolates the RF path with a deliberate 200 ms delay (ride-through for switch bounce on a moving gate). Logic. X_2oo3_B(GADC-04,05,06) & SIF-07_OUTPUT → SIF-13_TRIP. Test. GADC-04 = 0 (no trip) then GADC-05 = 0 → SIF-13_TRIP → 0; after ~200 ms `CON-06/07/08:EN → 0` (and CON-09:EN, same group, p.267).

### SIF-02 — Service Access Door (p.297)

Concept. The service door is a second personnel entry with its own switch sets: a 2oo3 triple plus a 1oo2 pair (different door leaf), OR'd — either voted-open isolates the source. Logic. (X_2oo3_B(SADC-01,02,03) & SADC-04 & SADC-05) & SIF-07_OUTPUT → SIF-02_TRIP, also publishes SERVICE_DOOR_CLOSED. Test A (2oo3): SADC-01 = 0, then SADC-02 = 0 → trip → CON-01/02/03 → 0. Test B (1oo2): baseline, then only SADC-04 = 0 → trips alone.

### SIF-03 — Current & Voltage monitoring / door-unlock permit (p.303)

Concept. Doors may only unlock when the source is provably dead. "Dead" is measured two independent ways — current (IT) and voltage (VT), each 2oo3 — and must hold for 20 s (DOOR_UNLOCK_DELAY) before the Open/Reset button is even enabled. Then a deliberate human action (OR-01) opens up. This prevents unlocking on a momentary dip or a single faulty transmitter. Logic. Two voters X_2oo3_B(IT-01..03_TRIP), X_2oo3_B_1(VT-01..03_TRIP), RS latch, TON(PT = T#20s), R_TRIG(OR-01), OR-01:LED = "press me now" lamp → BL10E-SIF-03_OUTPUT. Test.

| Step | Force (Option A: the _TRIP flags) | Expect |
| --- | --- | --- |
| 1 | IT-01_TRIP = 1, IT-02_TRIP = 1 | current voted present (2oo3) → locks SOL-0x held 1, OR-01:LED = 0 |
| 2 | IT-01_TRIP = 0, IT-02_TRIP = 0, wait <20 s | OR-01:LED still 0 (20 s confirm running) |
| 3 | wait ≥20 s | OR-01:LED → 1 |
| 4 | pulse OR-01 | SIF-03_OUTPUT permits unlock; SFC heads to OPEN (60 s RESET_DELAY) |
| 5 | repeat 1–4 with `VT-01_TRIP/VT-02_TRIP` | identical behaviour via the voltage voter |

*(Option B users: drive the same steps with field values instead — IT-0x = 12.0 for present, 3.9 for dead — after editing the setpoints.)*

### SIF-05 — Search Start permissive (p.344)

Concept. A search may only begin when one authorised person (card + both keys) starts it with the service doors already shut and no beam-off latched. Every condition is a hard gate — remove any one and the search must refuse to start. This is your negative-test set. Logic. SCR-01 & KEY-01 & KEY-02 & SADC-04 & SADC-05 & X_2oo3_B(SADC-01..03) & SIF-08_TRIP → SIF-05_START (single AND gate — beautifully readable on p.344). Test. From baseline, for each row force the one item, then SCR-01 = 1:

| Blocked by | Force | Expect |
| --- | --- | --- |
| (control) nothing | just SCR-01 = 1 | SIF-05_START → 1, STATUS-START_SEARCH → 1 ✔ |
| key 1 out | KEY-01 = 0 | SIF-05_START stays 0 — no search |
| key 2 out | KEY-02 = 0 | stays 0 |
| service door open | SADC-04 = 0 | stays 0 |
| BOB latched | press+release a BOB, no reset | stays 0 until …BOB:RESET pulsed |

### SIF-06 — Search Complete proof (p.350)

Concept. The final search button only counts if every general door and gate is shut at that instant — otherwise someone could still slip in. Logic. X_2oo3_B(GADC-01..03) & X_2oo3_B_1(GADC-04..06) → SIF-06_TRIP (+ GENERAL_ACCESS_DOOR_CLOSED, and SIF-06_OUTPUT which sets the door-lock solenoid latch in Logic p.267). Test. During the walkthrough (§4) at step ASB4: force GADC-01 = 0 and GADC-02 = 0, pulse ASBF-01 → T7 refused (SIF-06_TRIP = 0, no STANDBY). Close the doors, pulse ASBF-01 again → STANDBY.

### SIF-07 — Beam-On permissive / lock feedback + 180 s (p.356)

Concept. Locked doors must be proven, not assumed: the *feedback* switches on each lock (not the command) gate the beam permit, and a 180 s "radiation imminent" dwell (BEAM_DELAY_TIMER) gives anyone inside a last chance to hit a beam-off button. Command ≠ confirmation — core safety idea. Logic. GADL-01 & GADL-02 & GADL-05 & GADL-04 & SADL-01 & SADL-02 & SIF-06_TRIP/_OUTPUT → TON(BEAM_DELAY_TIMER_TIME = 180 s) + RS → BEAM_DELAY_TIMER_END / SIF-07_OUTPUT. Test (positive). In §4 after T7: force the six feedbacks GADL-01/02/04/05, SADL-01/02 = 1 (simulating the mechanisms), wait 180 s → BEAM_DELAY_TIMER_END → 1, SIF-07_OUTPUT → 1. Test (negative). Repeat the walkthrough but leave `SADL-02 = 0` (one lock fails to engage). Expect: SIF-07_OUTPUT stays 0, no BEAM_ON ever — this is the test v1 of this document was missing.

### SIF-08 — Beam-Off Buttons (p.363)

Concept. The emergency stop. Eight dual-channel buttons, any one (1oo7) kills source *and* RF (two independent hazard paths = "defence in depth") and unlocks the doors so people can escape. It latches: releasing the button must never restart a hazard — only a deliberate, separate reset may clear it. Logic. Per button :A & :B; all buttons into an RS latch; reset = …-BOB:RESET; output SIF-08_TRIP (permissive). Test.

| Step | Force | Expect |
| --- | --- | --- |
| 1 | BOB-03:A = 0 and BOB-03:B = 0 | SIF-08_TRIP → 0; CON-01,02,03,06,07,08,09,10:EN → 0; locks SOL-0x → 0 (instant escape unlock, C&E footnote a) |
| 2 | restore both = 1 | SIF-08_TRIP stays 0 (latched) |
| 3 | pulse …-BOB:RESET | SIF-08_TRIP → 1 (hazard still off — needs new search) |
| 4 | repeat for buttons 1,2,4,5,6,7,8 | each alone trips (1oo7) |

### SIF-09 — Radiation monitors (p.368)

Concept. Two independent monitors, each giving accumulated dose (RDMND) and dose-rate (RDMNR) signals; 1oo2 because a real radiation alarm must never be out-voted. Latched, with its own reset, so a transient can't self-clear. Logic. (RDMND-01, RDMNR-01, RDMND-02, RDMNR-02) → 1oo2 gating → RS latch, reset …-RDMN:RESET → SIF-09_TRIP. Test. Force RDMND-01 to trip → SIF-09_TRIP → 0 → CON-01/02/03 → 0 (+ CON-10, latch group). Restore; still tripped; pulse …-RDMN:RESET → clears. Repeat with RDMNR-01, RDMND-02, RDMNR-02.

### SIF-10 — Electron-Source Enable Key (p.372)

Concept. A physical key (kept by the responsible person) is the authorisation to have the source/RF energisable at all. 1oo1 — no voting, the key is the law. A returned key is not a reset. Logic. KEY-03 → SIF-10_TRIP; gates CON-04/05 (with SIF-08) and the RF group (p.267). Test. From BEAM_ON: KEY-03 = 0 → SIF-10_TRIP → 0 → CON-04/05:EN → 0 (watch which RF contactors follow on p.267). KEY-03 = 1 → hazard stays off; recover only via new search.

### SIF-11 — Oxygen monitoring (p.377)

Concept. Cryogenic nitrogen can displace air; any one of four monitors reading low (1oo4 — maximum sensitivity, people pass out without warning) closes the LN₂ supply valve and drives the O₂ alarm state. Latched + own reset. Logic. OXMON-01..04_TRIP → ≥1 → RS latch, reset …-GAS:RESET → SIF-11_TRIP → SDVLN2, O₂ beacons (SP01/02-04..07), indicators. Test (Option A). OXMON-02_TRIP = 1 → SDVLN2 → 0 (valve shut), O₂ beacons to alarm. OXMON-02_TRIP = 0 → still latched. Pulse …-GAS:RESET → clears. Repeat each monitor (1oo4). *(Option B: use OXMON-02 = 18.0 / 20.9 after editing the setpoint.)*

__________________________________________________

## 4 · The search walkthrough (SIF-04 SFC, pp.326–338) — full pass to BEAM_ON

Concept. The SFC is the choreography that proves "nobody inside": one person sweeps the hutch pressing buttons in a fixed deep-to-door order. Two timing layers police it: the overall 180 s MAX_SEARCH_TIME, and per-step Transition_Timers (p.285) with Min 5 s / Max 60 s — too fast means you didn't actually look (or two people are gaming it), too slow means lost control of the area. The light curtain counts entries: exactly one pass in, then any further beam break during T2–T6 aborts.

Watch list: STATUS-OPEN_READY/-START_SEARCH/-HUTCH_ENTERED/-ASB1..4/ -STANDBY/-BEAM_ON, SEARCHED_AND_LOCKED, SEARCH_TIMER_RUNNING, SEARCH_TIME_EXCEEDED, LIGHT_CURTAIN_MONITORING, BEAM_DELAY_TIMER_END, ABORT_SEARCH, Tx-Ty_TIMER_TRIP_LOW/HIGH, SIF-05_START, SIF-06_TRIP, SIF-07_OUTPUT.

| # | You force | State advances to | Also watch |
| --- | --- | --- | --- |
| 0 | §1 baseline | STATUS-OPEN_READY = 1 | — |
| T1 | SCR-01 = 1 | START_SEARCH | SIF-05_START → 1; SOL-05/06 → 1 (service locks); force `SADL-01/02 = 1` (feedback follows); ANNRES = 1; 180 s timer runs |
| — | wait ≥5 s | — | T1-T2_TIMER running |
| T2 | pulse LCRx-01: 1→0→1 | HUTCH_ENTERED | after 2 s LIGHT_CURTAIN_MONITORING → 1 |
| — | wait ≥5 s | — | — |
| T3 | pulse ASB-01: 0→1→0 | ASB1 | ASB-01:LED = 1 |
| — | wait ≥5 s | — | — |
| T4 | pulse ASB-02 | ASB2 | ASB-02:LED = 1 |
| — | wait ≥5 s | — | — |
| T5 | pulse ASB-03 | ASB3 | ASB-03:LED = 1 |
| — | wait ≥5 s | — | — |
| T6 | pulse ASB-04 | ASB4 | ASB-04:LED = 1; LC monitoring ends |
| — | wait ≥5 s | — | — |
| T7 | pulse ASBF-01 (needs SIF-06_TRIP = 1) | STANDBY | SOL-01..04 → 1; force `GADL-01/02/04/05 = 1` ← *without this the sequence stalls here*; ANNSTB = 1; BLUEL-0x = 1; SEARCHED_AND_LOCKED → 1; beacon SP02 |
| — | wait 180 s | — | BEAM_DELAY_TIMER_END → 1, SIF-07_OUTPUT → 1 |
| T8 | (transition fires) | BEAM_ON | ANNBON = 1; BONI-01:A/:B = 1; `CON-0x:EN → 1`; white lights pulse (TP T#1s, p.337); KEY-01:SOL releases |

### 4.1 Abort tests (each must land in an `*_ABORTED` path / back toward OPEN)

| Abort | Provoke | Watch |
| --- | --- | --- |
| Out of order | at START_SEARCH pulse ASB-01 before LCRx | ABORT_SEARCH → 1 |
| Too fast | next step <5 s after previous | that Tx-Ty_TIMER_TRIP_LOW → 1 → abort |
| Too slow | sit in a step >60 s | Tx-Ty_TIMER_TRIP_HIGH → 1 → abort |
| Whole search >180 s | dawdle overall | SEARCH_TIME_EXCEEDED → 1 → abort |
| Light curtain re-break | during T3..T6 pulse LCRx-01 = 0 | abort while LIGHT_CURTAIN_MONITORING = 1 |
| Key pulled mid-search | KEY-01 = 0 (or KEY-02) before T7 | abort |
| Door mid-search | GADC-01 = 0 + GADC-02 = 0 | abort |
| Door at the final button | doors open at ASB4, pulse ASBF-01 | T7 refused (SIF-06_TRIP = 0) |

### 4.2 KEY-01 release behaviour (C&E footnote k / p.337)

Concept. KEY-01 is trapped (solenoid) during the search; at BEAM_ON the program releases it so it can be carried to the rack position (KEY-03). Removing it *before* the search completes must abort; removing it *after* release must not. Test. (a) Mid-search KEY-01 = 0 → abort (done in 4.1). (b) Reach BEAM_ON, watch KEY-01:SOL → 0 (released).

__________________________________________________

## 5 · The no-silent-restart latch (`Logic` p.267) — the most important test

Concept. After any trip, clearing the cause must not re-energise the hazard. The contactor logic AND's the SIF permissives with SEARCHED_AND_LOCKED & STATUS-BEAM_ON through RS latches. The page carries this exact comment:

*"If Search and Locked is broken, contactors 01, 02, 03, 06, 07, 08, 09 & 10 can not start again when tripped without going through a new search sequence."*

Verified groupings (p.267):

| Latch group | Contactors | Gated by |
| --- | --- | --- |
| Source group | CON-01, 02, 03, 10 | SIF-08 & SIF-01 & SIF-02 & SIF-09 + SEARCHED_AND_LOCKED & STATUS-BEAM_ON |
| Gate/RF group | CON-06, 07, 08, 09 | SIF-13 (+ customer interlock CUST-01 path) |
| Key group | CON-04, 05 | SIF-10 & SIF-08 |
| Door locks | SOL-01..06 | set by R_TRIG(SIF-06_OUTPUT) (T1: service / T7: general), reset by STATUS-OPEN or SIF-08 trip |

Test. Reach BEAM_ON → trip anything (e.g. one BOB) → clear it → reset → confirm every contactor in the latch groups stays 0 → only a complete new §4 walkthrough re-energises. Do this once with a door trip and once with a BOB trip.

__________________________________________________

## 6 · The alarm/diagnostic layer (pp.162–174) — monitoring, not tripping

Concept. Around the SIFs sits a non-tripping layer that tells the operator *why* something happened: per-channel _ALARM flags (e.g. IT-0x_TRIP through a TOF with E_SOURCE_TIME = 120 s, p.172), dual/triple-channel discrepancy alarms (pp.171, 173–174), and system diagnostics (PSU/fan/ switch faults on DI-007, forcing-active flags, module errors — Diagnostics pp.241–250). These drive annunciation only. Test (sample). Force IT-01 = 12 for >120 s → IT-01_ALARM sets; restore → alarm clears after the TOF. Force GADC-01 = 0 alone for > voter DT → X_2oo3_B.Dev discrepancy flag (door switch disagreement = maintenance alarm).

__________________________________________________

## 7 · Coverage matrix (tick when done)

| # | Logic under test | Where | Concept proven |
| --- | --- | --- | --- |
| 2.1 | Analogue channel → _TRIP | pp.184–187 | scaling, setpoint, hysteresis |
| 2.2 | Dual-channel + discrepancy | pp.152–161, 171–174 | channel fault ≠ trip |
| 3 | SIF-01 door | p.291 | 2oo3 voting |
| 3 | SIF-13 gate | p.382 | 2oo3 + 200 ms |
| 3 | SIF-02 service door | p.297 | 2oo3 OR 1oo2 |
| 3 | SIF-03 IT+VT unlock permit | p.303 | 2×2oo3, 20 s confirm, manual OR |
| 3 | SIF-05 start permissive (5 negatives) | p.344 | AND-gate authorisation |
| 3 | SIF-06 search-complete proof | p.350 | doors closed at T7 |
| 3 | SIF-07 lock feedback + 180 s (pos+neg) | p.356 | command ≠ confirmation |
| 3 | SIF-08 BOB ×8 | p.363 | 1oo7, latch, escape unlock |
| 3 | SIF-09 radiation ×4 signals | p.368 | 1oo2, latch |
| 3 | SIF-10 key | p.372 | 1oo1, key ≠ reset |
| 3 | SIF-11 oxygen ×4 | p.377 | 1oo4, latch |
| 4 | Full SFC T1→T8 | pp.326–338 | the search choreography |
| 4.1 | 8 abort cases | pp.328–331 | both timing layers + monitors |
| 4.2 | KEY-01 trap/release | p.337 | search integrity |
| 5 | No-silent-restart latches | p.267 | the master safety property |
| 6 | Alarm layer | pp.162–174 | diagnosis vs protection |

## 8 · Known limits / honest notes

The analogue setpoints are fail-safe constants (OXMON_SP = 25.0,

IT/VT_SP = 0.0) — they cannot be forced; see §0.4 for the two ways to test   around them. The illustrative commissioning values used in the companion docs   (19.5 %O₂, >4 mA) come from the C&E, not from the program.

A few exact pin polarities are hard to read from the flattened export (e.g.

inverter bubbles): trust the live values in simulation, that's what it's for.

The SRS (TDI-PSS-SRS-0002) and SIF drawing (Dwg 1224211) are not in the

repo; for plant work reconcile against them (e.g. C&E note says 12 s where the   program implements DOOR_UNLOCK_DELAY = 20 s).

## 9 · Test record template

Test ref (matrix #): ____  Page: ____  Date: ____  Engineer: ____
Baseline OK (STATUS-OPEN_READY=1)?  Y / N
Setpoints forced (_SP values): ______________________________
Forced (tag = value): _______________________________________
Intermediate (tag : observed): ______________________________
Output (tag : observed): ____________________________________
Latched? reset used: ____________  New search needed?  Y / N
PASS / FAIL: ____  Notes: ___________________________________