# 4 · Program logic reference (the exact PLC code)

*This is the authoritative map of program.pdf (713 pages, HIMA SILworX export, SILworX v16, project C15862H-BL10E, configuration BL10E_HeXI [400], target HIMax, SIL 2). The searchable text is in data/program_extracted.txt (page markers ===== PAGE n =====).*

Convention in SILworX FBD text: & = AND, >=1 = OR, NOT/a bubble = invert. All effects are de-energise-to-trip: an output FALSE = tripped/safe; a SIF ..._OUTPUT/..._TRIP signal TRUE usually means healthy/permissive.

__________________________________________________

## 4.1 Page index (where everything lives)

| Pages | Area |
| --- | --- |
| 1–13 | Cover + Table of Contents |
| 14–16 | Project structure (POU tree, lists all SIFs) |
| 37–126 | Global Variables & cross-references (the master tag list + initial values, setpoints, timer presets) |
| 127–149 | Hardware (HIMax I/O module config) |
| 151–285 | C15862H_Lib — custom application logic (see 4.4) |
| 286–382 | The 12 SIFs (core safety logic) |
| 383–690 | X_Lib — HIMA standard blocks (voters, timers, limits) |
| 691–710 | Modbus TCP / OPC-UA / safeethernet |
| 711–713 | Watchpages (Global Forcing) |

## 4.2 The 12 SIFs (page → logic)

| SIF | Pages | Logic pg | Title |
| --- | --- | --- | --- |
| SIF-01 | 287–291 | 291 | General Access Door Closed |
| SIF-02 | 292–297 | 297 | Service Access Door Closed |
| SIF-03 | 298–303 | 303 | Current & Voltage Monitoring |
| SIF-04 | 304–338 | 326–338 | Search Sequence (SFC) |
| SIF-05 | 339–344 | 344 | Search Start |
| SIF-06 | 345–350 | 350 | Search Complete |
| SIF-07 | 351–356 | 356 | Beam On (permissive) / lock feedback |
| SIF-08 | 357–363 | 363 | Beam Off Buttons |
| SIF-09 | 364–368 | 368 | Radiation Monitoring |
| SIF-10 | 369–372 | 372 | Electron Source Enable |
| SIF-11 | 373–377 | 377 | Oxygen Monitoring |
| SIF-13 | 378–382 | 382 | General Access Gate Closed |

*(No SIF-12 in the PLC — the C&E "SIF 11 & 12" both refer to the oxygen function.)*

## 4.3 Per-SIF logic (plain English)

SIF-01 (p.291) — GADC-01/02/03 voted 2oo3 (X_2oo3_B), AND'd with

SIF-07_OUTPUT (beam-on conditions). Output SIF-01_TRIP → contactors   CON-01/02/03 via the central Logic POU.

SIF-02 (p.297) — SADC-01/02/03 2oo3 OR SADC-04/05 (1oo2 pair),

AND'd with SIF-07_OUTPUT. Outputs SIF-02_TRIP, SERVICE_DOOR_CLOSED.

SIF-03 (p.303) — "is the beam actually off?" + door-unlock permit.

Current IT-01/02/03 2oo3 and Voltage VT-01/02/03 2oo3. An RS latch + a   TON DOOR_UNLOCK_DELAY = T#20s + R_TRIG on Open/Reset pushbutton   OR-01 and key KEY-01: the Open/Reset button only becomes live once both   2oo3 votes read zero for ≥20 s, then OR-01:LED lights. (The C&E footnote   mentions 12 s from the SRS; the implemented value is 20 s.)

SIF-04 (pp.326–338) — the SFC (see 4.5).

SIF-05 (p.344) — Search-start permit (T1): SCR-01 card AND KEY-01

AND KEY-02 AND service-door 2oo3 AND 1oo2 pair AND SIF-08 healthy →   SIF-05_START.

SIF-06 (p.350) — Search-complete: general door 2oo3 AND gate 2oo3 →

SIF-06_TRIP (the T7 proof that all doors are shut).

SIF-07 (p.356) — master beam-on permissive: door-lock confirmation

switches GADL-01/02/04/05 + SADL-01/02 + SIF-06, with a TON   BEAM_DELAY_TIMER = T#180s and an RS latch → SIF-07_OUTPUT. Consumed by   SIF-01/02/13 and the SFC.

SIF-08 (p.363) — eight dual-channel beam-off buttons BOB-01…08

(:A/:B AND'd per button), 1oo7 (any one), RS-latched, reset   IOC-01-BOB:RESET. Drops CON-01,02,03,06,07,08,09,10 and instantly unlocks.

SIF-09 (p.368) — RDMND-01/02 (dose) + RDMNR-01/02 (rate), 1oo2,

RS-latched, reset IOC-01-RDMN:RESET. Drops CON-01/02/03 and CON-09.

SIF-10 (p.372) — KEY-03 rack key (1oo1) → SIF-10_TRIP; gates CON-04/05

(and CON-09/10 RF). Combined with KEY-01 (ZCP) elsewhere.

SIF-11 (p.377) — OXMON-01…04, 1oo4, RS-latched, reset

IOC-01-GAS:RESET. Closes LN₂ valve SDVLN2, switches O₂ beacons/indicators   IND-01/02/03 and speaker SP02.

SIF-13 (p.382) — gate GADC-04/05/06 2oo3, AND'd with SIF-07_OUTPUT →

drives CON-06/07/08 with a 200 ms delay. (Added in Rev 4.0 to split gate   from door.)

## 4.4 The central `Logic` POU (pp.267–269) — cause → effect

This is the block that actually drives the outputs:

Contactors (p.267): combine the SIF trips per group and RS-latch them so

that CON-01,02,03,06,07,08,09,10 cannot re-energise after a trip without a   fresh full search (the SEARCHED_AND_LOCKED latch). This is why, in the   simulator, you must re-search after any door/BOB trip.

Door/gate solenoids `SOL-01…06`: service doors lock when the search

*starts*; general doors/gate lock when the *final* button is pressed; all   unlock when status = Open or SIF-08 is active.

Lights/annunciators/sounders (pp.268–269): white light vs blue light

interlock, annunciator OPEN/RESTRICTED/STANDBY/BEAM-ON, key solenoids, speaker   programs, O₂ beacon.

## 4.5 The Search SFC + Transition_Timer (SIF-04)

Layer 1 — the SFC steps (Modbus bit in brackets): Open (0) → Open-Ready (1) → Search_Start (2) → Hutch_Entered (3) → ASB1 (4) → ASB2 (5) → ASB3 (6) → ASB4 (7) → Standby (8) → Beam On (9); with abort branches Search_Aborted, Start_Aborted, Run_Aborted. (Modbus bit 10 = SEARCHED_AND_LOCKED — a convenient state handle for an HMI.)

Layer 2 — six `Transition_Timer` instances (FB defined p.285), one per gap T1-T2 … T6-T7. Each takes Start_Timer, Min_Time, Max_Time, Reset and outputs Below_Min_Time (→ _TRIP_LOW), Exceed_Max_Time (→ _TRIP_HIGH) and Transition_Time (ms). All six use `Min = T#5s`, `Max = T#60s` and are reset by STATUS-START_SEARCH.

## 4.6 Verified timing constants (Global Variables)

| Tag | Value | Purpose |
| --- | --- | --- |
| MAX_SEARCH_TIME | 180 s | overall search watchdog |
| T1-T2…T6-T7_TIMER_MIN | 5 s | min per-step time (faster ⇒ abort) |
| T1-T2…T6-T7_TIMER_MAX | 60 s | max per-step time (slower ⇒ abort) |
| BEAM_DELAY_TIMER_TIME | 180 s | Standby → Beam-On dwell |
| RESET_DELAY_TIMER | 60 s | delay returning to Open |
| DOOR_UNLOCK_DELAY | 20 s | SIF-03 zero-source confirm |
| LIGHT_CURTAIN_DELAY | 2 s | LC monitoring arm delay |
| gate/RF contactor delay | 200 ms | CON-06…10 trip delay |

## 4.7 Function-block types used

Voters: X_2oo3_B (sum of 3 BOOLs ≥ 2; also a discrepancy Dev output) —

used in SIF-01/02/03/05/06/13. The higher votes 1oo4/1oo7/1oo2 in SIF-08/09/11   are done with plain &/>=1 gating + RS latches, not voter blocks.

Analogue trips: X_LimH (high trip) and X_LimL (low trip), each with

hysteresis, trip-delay DT, Inhibit, Reset, Ch_ok. X_Hx_AI scales raw   HIMax counts to engineering units.

IEC blocks: TON (on-delay), TP (pulse), RS/SR (latches),

R_TRIG/F_TRIG (edge detect), MOVE, LIMIT, comparators.

Custom: Transition_Timer, 2In_/3In_Discrepency_Alarm.

## 4.8 Analogue setpoints (as exported)

| Signal | Block | Setpoint | Hyst |
| --- | --- | --- | --- |
| OXMON-01…04 (O₂) | X_LimL (low) | 25.0 (full-scale placeholder; commissioned value ≈ 19.5 %) | 0.2 |
| IT-01…03 (current) | X_LimH (high) | 0.0 (placeholder) | 0.2 |
| VT-01…03 (voltage) | X_LimH (high) | 0.0 (placeholder) | 0.2 |
| RDMN dose/rate | digital trip contacts (>4 mA per C&E) | — | — |

Each AI channel also exposes _OC (open-circuit), _SC (short-circuit) and _CH_OK diagnostics. The simulator uses 19.5 %O₂ and 4 mA as illustrative trip points — adjust in data/cause_effect.json to match the commissioned values.

## 4.9 Things worth knowing for support work

Latching / "no silent restart": trips and the search-complete state are RS

latches. After any trip you need an explicit reset *and* (for the hazard) a   fresh search. Modelled exactly in the simulator.

Resets are separate: IOC-01-BOB:RESET (SIF-08), …-RDMN:RESET (SIF-09),

…-GAS:RESET (SIF-11), Open/Reset OR-01 (SIF-03, only live after the 20 s   zero-source confirm — watch OR-01:LED).

Dual-channel devices (BOB, BLUEL, ANNUN, BONI, BOBI) are read as :A/:B

pairs and AND'd; discrepancy-alarm blocks give first-up annunciation.

No application bypass/maintenance-override block exists — overrides are done

only via SILworX Global Forcing (Watchpages pp.711–712). (If you saw an   LXS_BYPASS block elsewhere, that belongs to a *different, unrelated* project   that used to be in this repo's history — not B10E.)

Comms: Modbus TCP slave on port 502 exposes the SFC state bits and the

oxygen readings; an OPC-UA server mirrors most tags. Good HMI/SCADA handles.

Naming wart: the PLC tag database uses BL10E-PS-… while the SIF block

names and the C&E "EPICS/DCS Tag" use B10E-PS-…. Same devices. Door locks   appear twice: as outputs SOL-01…06 and as feedback inputs GADL-/SADL-.

*External drawings referenced by the C&E but not in this repo:* SRS TDI-PSS-SRS-0002, SIF logic Dwg 1224211, layout Dwg 1212854.