# 5 · How to use the simulator

*No installation. Pure Python 3 standard library. Nothing connects to real plant.*

__________________________________________________

## 5.1 The three programs

| Command | What it does |
| --- | --- |
| python3 scenarios.py | Runs 7 worked examples as a self-test (PASS/FAIL). Run this first — if it is 7/7, the model matches the Cause & Effect matrix. |
| python3 pss_cli.py | Interactive — you type commands, it shows the live state. This is the one you'll use to learn and to rehearse the test procedure. |
| pss_sim.py | The engine (imported by the other two). Read it to see the logic in ~350 commented lines. |

## 5.2 Reading the status panel

After every command the CLI prints a panel. Top line = HUTCH STATE (the SFC step) with the search/step timers. Then:

inputs: card, keys, beam request; the open-vote counts for each door group

(e.g. Gen DOOR (grp A 2oo3) open-votes 1/3); current/voltage votes →   radiation_present; oxygen/BOB/RDMN counts and latch state.

outputs (ON = energised): e-source contactors C01..C05, RF C06..C10,

door locks, annunciator sign, blue lights/indicators, beacons, LN₂ valve, O₂   lights, and the Open/Reset LED.

Remember ON = energised: for a contactor that means *hazard powered*; for a lock it means *door locked*; lowercase/off = the safe state.

If something has tripped, a red TRIPS: line lists the reasons. If a search was aborted, a yellow last abort: line tells you why.

## 5.3 Command reference

| Command | Example | Meaning |  |
| --- | --- | --- | --- |
| search | search | auto-run a full valid search → BEAM_ON (does the waits for you) |  |
| card / start | card | insert the search card (begins T1 if keys/doors OK) |  |
| lcrx | lcrx | light-curtain single pass-through (T2) |  |
| asb1…asb4 | asb1 | press an area-search button (T3…T6) |  |
| asbf | asbf | final search button (T7 → STANDBY) |  |
| enable | enable | operator "enable beam" request (needed for T8) |  |
| wait <s> | wait 10 | let time pass — needed between search steps (≥5 s) |  |
| `door <tag> open\ | close` | door GADC01 open | move a door switch (GADC01-06, SADC01-05) |
| bob <n> [up] | bob 3 / bob 3 up | press/release beam-off button n (1-8) |  |
| `key <K> on\ | off` | key KEY03 off | turn an enable key (KEY01/02/03) |
| ai <tag> <val> | ai OXMON02 18 | set an analogue input (OXMON01-04, IT01-03, VT01-03) |  |
| lc | lc | extra light-curtain interrupt (aborts a monitored search) |  |
| reset <R> | reset BOB | press a reset PB (BOB / RDMN / GAS / OR) |  |
| state | state | reprint the panel |  |
| log | log | last 15 internal events (advances/aborts) |  |
| help / quit |  | help / exit |  |

## 5.4 Five things to try (10 minutes)

Happy path: search → watch it reach BEAM_ON with contactors energised.

E-stop & latch: from BEAM_ON, bob 1 (everything trips), bob 1 up

(stays tripped), reset BOB, then search again to recover.

2oo3 voting: from BEAM_ON, door GADC01 open (no trip), `door GADC02

open` (trips). One switch is tolerated, two are not.

Oxygen: ai OXMON03 17 → LN₂ valve closes, O₂ lights red; `ai OXMON03

20.9 + reset GAS` to clear.

Break a search: card, wait 2, lcrx → aborts "too fast"; or card,

wait 10, asb1 → aborts "out-of-order".

## 5.5 Changing the data

Everything the engine knows lives in data/cause_effect.json:

setpoints (e.g. oxygen 19.5 %, current/voltage 4 mA),

voting schemes per group,

the timing constants,

which causes/effects belong to which SIF.

Edit that file and re-run scenarios.py — no code change needed for setpoints or voting. The exact PLC values are in docs/04_PROGRAM_LOGIC_REFERENCE.md §4.6/4.8.

## 5.6 Honest limits of the model (so you don't over-trust it)

This is a teaching model of the logic, not the certified PLC code. In particular:

It is a clean state-machine; the real PLC also runs diagnostics, channel

OC/SC fault handling, discrepancy alarms, and dual-CPU voting that are not   modelled here.

Analogue behaviour is a simple setpoint compare (no scaling curve / hysteresis

detail).

The light-curtain "single pass vs re-interrupt" and the per-step 5–60 s windows

are modelled in spirit; confirm exact edge-cases against program.pdf.

Timing is event-driven via wait/tick, not real-time.

For anything that affects real plant, the C&E matrix, the SRS and `program.pdf` are the authority — this simulator is for understanding and rehearsal.