# B10E "HeXI" Personnel Safety System — study & test pack

A set of clear, plain‑English documents to help you understand the safety logic in this repository and test it step by step in HIMA SILworX offline simulation. Written for a junior technical‑support engineer.

### 👉 New here? Read `docs/00_START_HERE.md` first. It is a single guided document — system, code, and how to simulate it — and it is all you need to understand the whole thing. The other documents are deep reference you can dip into later.

What is this system? The three source files in this repo describe the Personnel Safety System (PSS) for the Diamond Light Source B10E "HeXI" beamline. A beamline has a shielded room called a hutch. Inside the hutch there are things that can hurt a person: radiation, RF (radio‑frequency) power, and low oxygen from cryogenic gas. The PSS is an independent safety PLC (a HIMax, from HIMA) whose one job is simple: a person and the hazard must never be in the hutch at the same time. Before the hazard can be switched on, a trained person must walk through and "search" the hutch to prove nobody is inside, then lock the doors. While the hazard is on, if anything unsafe happens (a door opens, a stop button is pressed, radiation is detected, oxygen drops) the PLC removes the hazard in a fraction of a second. The system is rated SIL 2 (a measure of how trustworthy the protection must be).

__________________________________________________

## The three source files (already in this repo)

| File | What it is | Plain meaning |
| --- | --- | --- |
| TDI-PSS-B10E-C&E Cause & Effect Matrix (14-04-26) v4.0.pdf | Cause & Effect (C&E) matrix | A big grid that says which sensor (cause) switches off which output (effect). |
| C15862H-HeXI-IOS.xlsx | I/O schedule (IOS) | A list of every signal and which physical PLC terminal it is wired to. |
| program.pdf | SILworX program printout | The actual PLC program: the safety functions, the search, all the timers. |

These documents turn those three files into something you can read and test.

__________________________________________________

## What's in this folder

B10E-PSS-Simulator/
├── README.md                 <- you are here
├── docs/
│   ├── 00_TAG_GLOSSARY.md             <- what every signal name (tag) means
│   ├── 01_SYSTEM_AND_CAUSE_EFFECT.md  <- the big picture + how to read the C&E
│   ├── 02_SEARCH_SEQUENCE.md          <- the hutch search, step by step
│   ├── 03_PROGRAM_LOGIC_REFERENCE.md  <- the exact PLC logic (pages, blocks, timers)
│   ├── 04_SILWORX_OFFLINE_TEST_PROCEDURE.md  <- step-by-step test in SILworX
│   └── 05_CODE_WALKTHROUGH.md         <- the real PLC pages (screenshots) explained one by one
├── docs-word/                <- the same documents as Microsoft Word (.docx) files
└── data/                     <- reference data pulled out of the three source files
    ├── io_map.json           <- the I/O list as data
    ├── ios_csv/              <- the I/O schedule, one sheet per file
    └── program_extracted.txt <- searchable text of all 713 program pages

Note: this pack is documentation only. There is no software to install and nothing here connects to real plant. You do the actual testing in your own HIMA SILworX offline simulation, following document 04.

__________________________________________________

## What each document tells you

| Document | What it tells you |
| --- | --- |
| 00 · Tag glossary | The full list of signal names (tags) and what each one means in plain words. Keep this open while you read the others. |
| 01 · System and Cause & Effect | The big picture: what the hutch hazards are, what a "cause" and an "effect" are, what de‑energise‑to‑trip means (OFF = safe), what voting means (e.g. "2 out of 3 sensors must agree"), and a one‑line summary of every safety function. Start here. |
| 02 · The hutch search | How the walk‑through search works, step by step (the stages the system moves through), the time limits, and what cancels a search. |
| 03 · Program logic reference | The exact PLC program: which page does what, how each safety function is built (which sensors, which voting, which timers), and the real set‑point and timer values. Use it as a look‑up. |
| 04 · SILworX offline test procedure | The main "do this" document: a step‑by‑step test you run in HIMA SILworX offline simulation. For each safety function it gives the tag to force, the signal to watch, and the result to expect. |
| 05 · Code walkthrough | A screenshot of each important PLC page taken straight from program.pdf, with a plain "how it works" explanation under each picture. Read it when you want to see the *actual* logic (the voters, AND gates, latches and timers) rather than a description of it. |

__________________________________________________

## How to use this pack

Read document 01 to understand what the system does and the key idea

(OFF = safe).

Keep document 00 (the glossary) open so every tag makes sense.

Read document 02 to understand the hutch search.

Open document 04 and your SILworX project side by side, and work through

the tests one safety function at a time. Use document 03 whenever you want    the exact page or block behind a test.

Everything you need to test the logic is in document 04, written so each step is clear: which signal to set, what to watch, and what should happen.